SECOND TERM SESSION

SUBJECT: PHYSICS   CLASS: SS2 TIME: 2HOURS

Instruction: Answer all questions in section A and B. (10 Marks)

- The velocity ratio of a plane inclined at an angle of θto the horizontal is                                                                                           (A) sin θ 	(B) tan θ  	(C) 1/sin θ 	(D) 1/tan θ

- The plane which is inclined at an angle of 30˚ to the horizontal has velocity ratio of                      (A) 2.00 		(B) 1.00  	(C) 0.87 		 (D) 0.50

- A block and tackle system has 6 pulleys. If the efficiency of the machine is 60%, determine its mechanical advantage.  (A) 12  	(B) 10  		(C) 3.6 		 (D) 1.8

- A screw jack has a pitch of 0.5cm and a tommy bar of length 20cm.The velocity ratio is (A) 40π		 (B) 40/π 	(C) 80π		 (D) 80/π

- A wheel and axle has radii of 50cm and 10cm respectively. If one-fifth of the work done by the effort is used in overcoming friction, an effort of 500N applied to the wheel will raise a load of 	(A) 2, 500N 	(B) 2,000N 	(C) 1, 000N 	(D) 500N

- A thermocouple works on the principle of (A) variation of e.m.f with temperature (B) variation of volume with temperature.  (C) variation of resistance with temperature (D) variation of pressure with temperature

- A constant volume gas thermometer works on the principle of (A) variation of e.m.f with temperature (B) variation of volume with temperature (C)variation of resistance with temperature (D) variation of pressure with temperature

- A liquid-in-glass thermometer works on the principle of (A) variation of e.m.f with temperature (B) variation of volume with temperature. (C) variation of resistance with temperature (D) variation of pressure with temperature

- The clinical thermometer differs from other mercury in glass thermometer because it has (A) a constriction (B) a narrow bore (C) a wide range (D) steel index

- The temperature of 45˚ is the same as  (A) 25˚F (B)  57˚F (C) 81˚F (D) 113 ˚F

- The lower fixed point correspond to a length of 20mm on the stem of  a thermometer  while the upper fixed point corresponds to 160mm.The temperature corresponding to a length of 48mm is  (A) 20.0˚ (B) 30.0˚C (C) 41,7˚C (D) 50.0˚C

- A platinum resistance thermometer has resistances of 5.25Ω and 9.75Ω at 0˚C and 100˚C respectively .When the resistance is 8.25Ω the temperature is                                                      (A) 63.6˚C (B) 66.7 ˚C  (C) 75.0˚C  (D) 84.6˚C

- This is the quantity of heat required to raise the temperature of an entire body by one degree rise in Temperature(1k)                                                                                                               (A) heat capacity (B) specific heat capacity (C) latent heat of fusion (D) thermal energy

- The quantity of heat needed to raise the temperature of a unit mass of a substance by 1K is the body’s                                                                                                                        (A) heat capacity (B) specific heat capacity (C) latent heat of fusion (D) thermal energy

- Which of the following cannot be used to measure the temperature of a substance? (A) Variation of pressure with temperature (B) Expansivity of liquid (C) Change in resistance of a conductor (D) Change in colour with temperature

- How much heat is given out when a piece  of iron of mass 50g and specific heat capacity 460Jkg-1K-1 cools from 85℃ to  250C ? (A) 1380J   (B) 1080J   (C) 1830J  (D) 3810J

- An electric heater of 60W is used to heat a metal block of mass 20kg for 5minutes.Calculate the specific  heat capacity of the metal block if the rise in temperature is 200C. (A) 40J/KgK 	(B) 45J/kgK  	(C) 50J/KgK 	(D)55J/kgK

- A tap supplies water at 260C while another supplies water at 820C. If a man wishes to bath with water at 400C the ratio of the mass of hot water to that of cold water is (A)   1:3 (B) 3:1 (C) 3:7 (D) 7:3

- A water fall is 500m high. If the water retains 65% of the heat generated at the end of the fall .Calculate the change in temperature due to the fall.(Specific heat capacity of water  = 4200J/KgK , g=10m/s2) (A) 3.30C (B) 2.10C (C) 0.20C (D) 1.20C

- On a hot and sunny day, the most comfortable colour of attire is                                              (A) Black    (B) white   (C) red (D) blue

- The property of a body to remain at rest or to continue to move on a straight line, is known as (A) force (B) impulse (C) momentum (D) inertia

- For elastic collision, (A) energy is doubled and momentum is halved (B) energy is conserved (C) momentum is conserved (D) kinetic energy and momentum are conserved

- The product of pressure (P) and volume (V) has the unit of (A) Force (B) power (C) impulse (D) work

- A wheel and axle of radii 800mm and 200mm respectively is used to raise a  body of weight 800N

by the applicationof 250N. Calculate the efficiency of the machine.

(A)    85%                (B)    80% (C)    60%                (D)   20%

- The radius of a wheel is 30.0cm and that of its axle is 6.0cm. Calculate the effort  required to lift a

load of 12.0N using this machine assuming 100%  efficiency. (A)   1.2N (B)  12.0N (C)   2.4N                   (D)  3.0N

- A machine of velocity ratio 5 is used in  lifting a load with effort of 500N. If the machine is 80%

efficient, determine the magnitude of the load.   (A)    2500N   (B)   2000N  (C)    900N          (D)   625N

- The plane which is inclined at an angle of 30˚ to the horizontal has velocity ratio of (A) 2.00  (B) 1.00  (C) 0.87  (D) 0.50

- A block and tackle system has 6 pulleys. If the efficiency of the machine is 60%, determine its mechanical advantage. (A)12  (B) 10  (C) 3.6  (D) 1.8

- A screw jack has a pitch of 0.5cm and a tommy bar of length 20cm.The velocity ratio is (A) 40π  (B) 40/π  (C) 80 π  (D) 80/π

- A wheel and the axle of a wheel and axle have radii 50cm and 10cm respectively. If one-fifthof the work done by the effort is used in overcoming friction, an effort of 500N applied to the wheel will raise a load of  (A) 2,500N	   (B) 2,000N 	(C)1,000N	(D)  500N

- A ball of mass 0.5kg moving at 10m/s collides with another ball of the same mass at rest. If the two balls move off together after impact. Calculate their common velocity (A) 0.2m/s  (B) 0.5m/s  (C) 5.0m/s  (D) 10m/s

- Which of the following equations for the efficiency of a machine is  correct? [ E = efficiency,V = velocity ratio,   M= mechanical Advantage (A) E = V/M x100% (B) E = input/output x 100%  (C) E = load distance/effort distance  x           100%   (D)    E = M/V x 100%

- A constant force of 5 N acts for 5 seconds on a mass of 5 kg initially at rest. Calculate the final momentum                                                                                               (A) 125 kgms ˉ1	(B) 25 kgms ˉ1	(C) 15 kgms ˉ1 (D) 5 kgms ˉ1

- Which of the following statements correctly defines a simple machine?   (A)   that can produce electric current (B)   which can only carry people from one place to another.(C) with which work can be done easily (D)   which changes the state of rest or  uniform motion  of an object

- A boy of weight 300N climbs to the top of a hill of height 20m. The work done by the boy against the force of gravity is (A) 6000J    (B) 600J (C)320J        (D)   15J

- Which of the following quanties has the same unit as energy?     (A) Power (B) Work (C) Force (D) Momentum

- A box is pulled a distance along a smooth horizontal floor by a force of magnitude F ,inclined at an angle θ to the horizontal.The work done is      (A)Fscosθ     (B)Fs cosecθ            (C)Fs sec θ(D)Fs sin θ

- A body initially at rest is accelerated at the rate of 0.2 m/s2 for 5s under a constant force of 50N.The work done on the body is                                                           (A) 10J  (B) 50J (C) 125J  (D) 250J

- A bullet of mass 40g is travelling at 300m/s strikes a target normally and penetrates a distance of 20cm in the target before coming to rest .The average force exerted on the bullet by the target is                                                         (A) 90N (B) 400N  (C ) 900N  (D) 9000N

- A car of mass 800kg initially at rest is accelerated at the rate of 4m/s2 .The kinetic energy of the car after 5 seconds is  (A) 6.40 X 103J (B) 2.56 X 104J (C) 1.60 X 105 (D) 6.40 X 105 J

ESSAY

1. A screw jack, 25% efficient and having a screw of pitch 0.4 cm is used to raise a load through a certain height. If in the process the handle turns through a circle of radius 40 cm, calculate the

(i) Velocity ratio of the machine

(ii) The Mechanical advantage of the machine (Take π= 3.14)

2. A piece of copper of mass 50g at  temperature 2500C is placed in a copper calorimeter of mass 70g containing 60g of water at temperature of 200C.Ignoring heat losses,what will be the final steady temperature of the mixture?

[Specific heat capacity of copper = 400J/Kgk,Specific heat capacity of water = 42000J/Kgk]

3(a) State newton’s first, second and third laws of motion   3marks

(b) Distinguish between elastic and inelastic collisions   2marks

4(a) Define efficiency of a machine             1mark

(b) Derive the formula that connectsefficiency of a machine ,mechanival advantage and velocity ratio      4marks

MARKING GUIDE FOR SECOND TERM

SUBJECT: PHYSICS    CLASS: SS2

SECTION A: OBJECTIVE TEST

| 1C | 2A | 3C | 4C | 5B | 6A | 7D | 8B | 9A | 10D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11A | 12B | 13A | 14B | 15D | 16A | 17B | 18A | 19D | 20B |

| 21D | 22D | 23D | 24B | 25C | 26B | 27A | 28C | 29C | 30B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31C | 32D | 33B | 34C | 35A | 36B | 37A | 38C | 39D | 40C |

(12mk each=20mks)

1 .i. velocity ratio, V.R =2πrp	1 mark

=2 X 3.14 X400.4= 628		1 mark

ii. Efficiency=M.A  V.RX 100% 	1 mark

MA =Efficiency X V.R100		1 mark

= 25 X 628100

= 157				1 mark

2.	McuCCU(tcu– t2) = McCc(t2 – tc) + MwCw(t2 – tw)	1 mark

0.050 x 400(250- t2) = 0.070 x 400(t2-20) + 0.060 x 4200(t2-20)

20(250- t2) = 28(t2-20) + 252(t2-20)		1 mark

5000- 20t2 = 28t2-560 + 252t2 - 5040

5000 + 560 + 5040 = 28t2  + 252t2 + 20t2	1  mark

10600  = 300t2					1 mark

t2 =   10600/30  =  35.330C		1 mark

3 (a) (i) Newton’s first law of motion:  The first law states that every object continues in its state of rest or uniform motion in a straight line unless acted upon by an external force. 1mark

(ii) Newton’s second law of motion: The rate of change of momentum of a body is directly proportional to the applied force and takes place in the direction in which the force acts.

force∝changeinmomentumtime       1mark

(iii)    Newton’s  third law of motion: It states that Action and Reaction are equal and opposite. Or to every Action there is an equal and opposite Reaction.  1mark

(b)  (i)Elastic collision: In an elastic collision both momentum and kinetic energy are conserved. This means that for two colliding bodies with masses m1 and m2  and initial velocities u1 and u2 and final velocities after collision V1 and V2 ,

M1U1 + M2U2 = M1V1 + M2V2,

12M1U12  +12 M2U22  =12 M1V12 +12M2V22  1mark

(ii)  Inelastic collision: In this case momentum is conserved but kinetic energy is not conserved. The energy lost        is usually converted to heat, sound or elastic potential energy.

M1U1 + M2U2 = M1V1 + M2V2,

12M1U12  +12 M2U22 ≠12 M1V12 +   12M2V22  1mark

4(a)

It also defined as the ratio of work output  of the machine to the total work input expressed in percentage.

Efficiency =   Work output    X  100%   1mark

Workinput

(b)  Efficiency (E)   = Workoutput       x  100%    1mark

Workinput

Efficiency (E) = Work done in load       x 100%

Work done in effort

Since Work = force x distance 1mark

Efficiency (E)  =     Load(L) x distance moved by load(dL)    x 100%   1mark

Effort(e) x distance moved by effort(de)

Therefore  E =L × dLe × de ×100

but MA=Le

and1VR=dLde

Therefore E =M.AV.R ×100% 1mark